Department of Computer Engineering

Faculty of Engineering

Khon Kaen University

### Experiment DSP-07:	FIR Filter Design Using Window Techniques

## Objectives:

- To determine specifications needed for FIR filter design.

- To learn how to design the FIR filters using window techniques.

- To learn how to implement the designed FIR filter by MATLAB.

## Introduction:

In order to process signals, we have to design and implement systems called filters. The filter design issue is influenced by such factors as the type of the filter or the form of its implementations. Two types of digital filters can be identified, FIR and IIR filters. FIR filters are characterized by finite-duration impulse response. IIR filters are characterized by infinite-duration impulse response. We will only study FIR filter design in this experiment. In the next experiment we will carry out a similar treatment for IIR filters.

A FIR filter is a recursive filter that has a transfer function of the form

H(z)=b0+b1z−1+⋯+bM−1z1−M=n=0M−1bnz−n	(1)

Hence the impulse response h(n)is

h(n)=bn,0≤n≤M−10,else 	(2)

and the difference equation representation is

y(n)=b0x(n)+b1x(n−1)+…+bM−1x(n−M−1)	(3)

Essentially, a FIR filter is designed based on specifications. Absolute and relative specifications of a lowpass filter are shown as

Rp



As

Decibels



P

S

|H()|

1

2



P

S



where 1 is passband ripple, Rp is passband ripple in dB, 2 is stopband ripple, As is stopband ripple in dB, p is passband edge frequency, and s is stopband edge frequency. The parameters given in the above two specifications are obviously related by

Rp=−20log101−δ11+δ1>0 (≈0)

(4)

As=−20log10δ21+δ1>0 (>>1)

Note: Unit of Rp and As is decibel (dB). By the meaning of attenuation, we always have the positive values of  Rp and As.

The basic idea behind the window design is to select an appropriate window function and an appropriate ideal filter that provides a linear-phase and causal FIR filter. Let w(n) and hd(n)be the window function and the impulse response of the ideal filter, respectively. The impulse response of the designed filter can be formed by

h(n)=hd(n)w(n) 	(5)

There are several window functions available. Each of those window functions has different characteristics in both time-domain and frequency-domain. We now briefly describe various well-known window functions.

- Rectangular Window This is the simplest window function but provides the worst performance from the viewpoint of stopband attenuation. It is defined by

	w(n)=1,0≤n≤M−10,otherwise 	(6)

- Bartlett Window Since the Gibbs phenomenon results from the fact that the rectangular window has a sudden transition from 0 to 1 (or 1 to 0), Bartlett suggested a more gradual transition in the form of a triangular window, which is given by

w(n)=2nM−1,0≤n≤M−122−2nM−1,M−12≤n≤M−10,otherwise	(7)

- Hanning Window This is a raised cosine window given by

w(n)=0.5∗1−cos2πnM−1,0≤n≤M−10,otherwise	(8)

- Hamming Window This window is similar to the Hanning window except that it has a small amount of discontinuity and is given by

w(n)=0.54−0.46*cos2πnM−1,0≤n≤M−10,otherwise	(9)

- Blackman Window This is also similar to the previous two but contains a second harmonic term and is given by

w(n)=0.42−0.5cos2πnM−1+0.08cos4πnM−1,0≤n≤M−10,otherwise		(10)

## Given transition width and minimum stopband attenuation of the desired filter, we can select suitable window type and widow size based on information in Table 1.

| Window Name | Transition Width  |  | Min. Stopband Attenuation |
| --- | --- | --- | --- |
|  | Approximate | Exact Values |  |
| Rectangular |  |  | 21 dB |
| Bartlett |  |  | 25 dB |
| Hanning |  |  | 44 dB |
| Hamming |  |  | 53 dB |
| Blackman |  |  | 74 dB |

###### Table 1. Summary of commonly used window function characteristics

- Kaiser window

(11)

where  = (M-1)/2 and I0 = the zeroth order Modified Bessel Function

If ωp, ωs, Rp, and As	are given then the following equations are needed for design.

Transition width=Δf=ωs−ωp2π

(12)

MATLAB provides several routines to implement window functions. A brief description of these routines is given below.

- w = boxcar(M) returns the M-point rectangular window function in array w.

- w = triang(M) returns the M-point Bartlett (triangular) window function in array w.

- w = hanning(M) returns the M-point Hanning window function in array w.

- w = hammimg(M) returns the M-point Hamming window function in array w.

- w = blackman(M) returns the M-point Blackman window function in array w.

Experiment:

- To design FIR filters based on the window technique, an ideal lowpass impulse response hd(n) is required. Therefore it is convenient to have a simple routine that creates hd(n) as shown below.

function hd = ideal_lp(wc, M)

% Ideal LowPass filter computation

% ----------------------------------------

% [hd] = ideal_lp(wc, M)

%   hd = ideal impulse response between 0 to M-1

%   wc = cutoff frequency in radians

%    M = length of the ideal filter

%

alpha = (M-1)/2;

n = [0:1:(M-1)]

M = n – alpha + eps;	% add smallest number to avoid divide by zero

hd = sin(wc*M) ./ (pi*M);

- To display the frequency-domain plots of digital filter, create and save the below function, which returns the magnitude response in absolute as well as in relative dB scale, the phase response, and the group delay response.

function [db, mag, pha, grd, w] = freqz_m(b, a)

% Modified version of freqz sunroutine

% ---------------------------------------------

% [db,mag,pha,grd,w] = freqz_m(b, a)

%  db = Relative magnitude in dB computed over 0 to pi radians

% mag = absolute magnitude computed over 0 to pi radians

% pha = Phase response in radians over 0 to pi radians

% grd = group delay over 0 to pi radians

%   w = 501 frequency samples between 0 to pi radians

%   b = numerator polynimial of H(z)   (for FIR: b=h)

%   a = denominator polynomial of H(z) (for FIR: a=[1])

%

[H,w] = freqz(b,a,1000,’whole’);

H = (H(1:1:501))’;

w = (w(1:1:501))’;

mag = abs(H);

db = 20*log10((mag+eps)/max(mag));

pha = angle(H);

grd = grpdelay(b,a,w);

% End of function

3. With the given specifications
			ωp=0.3π, ωs=0.4π, Rp=0.25 dB,and As=50 dB
Enter the following commands as design steps of the FIR lowpass filter using hamming window. Save the plots
>> wp = 0.3*pi; ws = 0.4*pi;

>> tr_width = ws-wp;

>> M = ceil(6.6*pi/tr_width)+1;

>> n = [0:1:M-1];

>> wc = (ws+wp)/2;

>> hd = ideal_lp(wc,M);

>> w_ham = (hamming(M))’;

>> h = hd .* w_ham;

>> [db,mag,pha,grd,w] = freqz_m(h,[1]);

% plots

>> subplot(1,1,1)

>> subplot(2,2,1); stem(n,hd); title(‘Ideal Impulse Response’)

>> axis([0 M-1 –0.1 0.3]); xlabel(‘n’); ylabel(‘hd(n)’)

>> subplot(2,2,2); stem(n, w_ham); title(‘Hamming Window’)

>> axis([0 M-1 0 1.1]); xlabel(‘n’); ylabel(‘w(n)’)

>> subplot(2,2,3); stem(n,h); title(‘Actual Impulse Response’);

>> axis([0 M-1 –0.1 0.3]); xlabel(‘n’); ylabel(‘h(n)’);

>> subplot(2,2,4); plot(w/pi, db); title(‘Magnitude Response in dB’);

>> grid

>> axis([0 1 –100 10]); xlabel(‘frequency in pi units’);

>> ylabel(‘Decibels’)

4. Find the value of M, Rp, and Asof the designed filter in the procedure (3)

5. Using the MATLAB function called filter, apply the filter designed in (3) to the input signal used in the previous experiment, which saved in the file lab4sig.mat. Then plot the input signal and the filtered output signal. Save the plot.

6. Design the FIR highpass filter using Kaiser window. Save the plots.
>> wp = 0.3*pi; ws = 0.4*pi; As = 50;

>> tr_width = (ws-wp)/(2*pi);

6.1 Write your own equation to compute the filter order M for the Kaiser window

>> M =

>> n = [0:1:M-1];

>> wc = (ws+wp)/2;

>> hd = ideal_lp(pi, M)-ideal_lp(wc,M);

6.2 Write your own equation to compute parameter  for the Kaiser window

>> beta =

6.3 Write your own equation to compute Kaiser window coefficients (see function besseli in MATLAB for help)

>> w_kaiser =

>> h = hd .* w_kaiser;

>> [db,mag,pha,grd,w] = freqz_m(h,[1]);

% plots

>> subplot(1,1,1)

>> subplot(2,2,1); stem(n,hd); title(‘Ideal Impulse Response’)

>> axis([0 M-1 –0.1 0.3]); xlabel(‘n’); ylabel(‘hd(n)’)

>> subplot(2,2,2); stem(n, w_ham); title(‘Hamming Window’)

>> axis([0 M-1 0 1.1]); xlabel(‘n’); ylabel(‘w(n)’)

>> subplot(2,2,3); stem(n,h); title(‘Actual Impulse Response’);

>> axis([0 M-1 –0.1 0.3]); xlabel(‘n’); ylabel(‘h(n)’);

>> subplot(2,2,4); plot(w/pi, db); title(‘Magnitude Response in dB’);

>> grid

>> axis([0 1 –100 10]); xlabel(‘frequency in pi units’);

>> ylabel(‘Decibels’)

Questions:

1. Given the specifications for a digital bandpass filter as below

lower stopband edge:ω1s=0.2π,As=60 dBlower passband edge:ω1p=0.35π,Rp=1 dBupper passband edge:ω2p=0.65π,Rp=1 dBupper stopband edge:ω2s=0.8π,As=60 dB

  - Sketch the relative specifications of the filter.

  - Design the filter using the Blackman window.

2. What is a built-in MATLAB function that can be used to design FIR filters? Explain how to use that function.

Nawapak Eua-anant and Rujchai Ung-arunyawee, 24 January 2549

Name:______________________________________________________________ID:________________

##### In class assignment DSP07

1. From This experiment, explain briefly advantages of FIR Filters.

2. For the rectangular, Blackman, Hamming, and Hanning windows with equal size, write down the name of the windows in the order based on width of the main lobe of their spectrum from the narrowest to the widest.

3. Which MATLAB commands in procedure 3 that are used to compute the number of points of the Hamming window?

4. What knowledge do you get from this lab?